Risk assessment

Company name: Kestrel Build Ltd – Assessment carried out by: Martin Hale

Date assessment was carried out: 7 October 2026 – Date of next review: 7 January 2027

Risk Assessment Table

| What are the hazards? | Who might be harmed and how? | What are you already doing to control the risks? | What further action do you need to take to control the risks? | Who needs to carry out the action? | When is the action needed by? | Done |
| --- | --- | --- | --- | --- | --- | --- |
| Falls from scaffold and roof edge; falling materials from scaffold and roof work | Roofers and labourers at risk of falls from the scaffold and roof edge; public on the shared driveway at risk from falling materials | Independent tied scaffold erected by Avonside Scaffolding on 1 October; handover certificate on site; weekly inspection by MH. Edge protection and toe boards fitted. | Brick guards on the top lift and a debris chute for the skip | Martin Hale | 9 October 2026 | no |
| Silica dust from disc cutter used for cutting blocks and paving | Operatives breathing silica dust; neighbours' gardens downwind | Cutter has a water suppression bottle; FFP3 masks issued and face-fit tested | Hire an on-tool extraction saw for internal cutting | Leanne Brooks (buyer) | 12 October 2026 | no |
| Cement and lime handling — cement burns and dermatitis | Labourers at risk of cement burns and dermatitis | Gloves and goggles issued, washing facilities in the welfare unit | No further action — existing controls are adequate | Not applicable | Not applicable | yes |
| Manual handling of block deliveries carried by hand to the rear | Labourers at risk of back injuries | Toolbox talk on lifting given on 5 October | Hire a powered barrow and brief the team on lifting | Martin Hale | 14 October 2026 | no |

Further Information

More information on managing risk: www.hse.gov.uk/simple-health-safety/risk/

Published by the Health and Safety Executive 10/19